GPT DSP Processor

Microarchitecture Specification

Revision 2.0

November 18, 2019

华夏芯（北京）通用处理器技术有限公司

General Processor Technologies Inc.

Revision history

| Revision | Author | Date | Description |
| --- | --- | --- | --- |
| 1.0 | TingYang | 2019-10-18 | Initial version |
| 2.0 | TingYang | 2019-11-18 | Filter accelerate |

CONTENT

1	PARAMETERS	1

1.1	HXGPTX ISA	1

1.2	FEATURES	1

1.3	SPR DEFINE	2

2	OVERVIEW	3

1.4	MEMORY HIERACHY	3

1.4.1	TCM	3

1.4.2	CACHE	4

1.4.3	PXI	4

1.4.4	AXI	4

1.5	INTERCONNECT	5

1.6	INTERFACE	6

3	MICRO-ARCHITECTURE	7

1.7	IFETCH	7

1.7.1	INTRODUCTION	7

1.7.2	OVERVIEW	7

1.7.3	INTERFACE	8

1.7.4	FUNCTIONS	8

1.7.5	CRITICAL PATHS	14

1.8	DISPATCH	15

1.8.1	INTRODUCTION	15

1.8.2	OVERVIEW	15

1.8.3	INTERFACE	17

1.8.4	FUNCTIONS	17

1.8.5	CRITICAL PATHS	50

1.9	INTEGER	51

1.9.1	INTRODUCTION	51

1.9.2	OVERVIEW	51

1.9.3	INTERFACE	52

1.9.4	FUNCTIONS	53

1.9.5	CRITICAL PATHS	60

1.10	DATXFER	61

1.10.1	INTRODUCTION	61

1.10.2	OVERVIEW	61

1.10.3	INTERFACE	62

1.10.4	FUNCTIONS	63

1.10.5	CRITICAL PATHS	65

1.11	DCACHE	66

1.11.1	INTRODUCTION	66

1.11.2	OVERVIEW	66

1.11.3	INTERFACE	67

1.11.4	FUNCTIONS	67

1.11.5	CRITICAL PATHS	69

1.12	VECTOR	70

1.12.1	INTRODUCTION	70

1.12.2	OVERVIEW	70

1.12.3	INTERFACE	70

1.12.4	FUNCTIONS	71

1.12.5	CRITICAL PATHS	76

1.13	WRITE BACK	77

1.13.1	INTRODUCTION	77

1.13.2	OVERVIEW	77

1.13.3	INTERFACE	78

1.13.4	FUNCTIONS	78

1.13.5	CRITICAL PATHS	81

1.14	TIMER	82

1.14.1	INTRODUCTION	82

1.14.2	OVERVIEW	82

1.14.3	FUNCTIONS	82

1.15	INTSPR	83

1.15.1	INTRODUCTION	83

1.15.2	OVERVIEW	83

1.15.3	INTERFACE	84

1.15.4	FUNCTIONS	84

1.15.5	CRITICAL PATHS	86

1.16	COREIO	87

1.16.1	INTRODUCTION	87

1.16.2	OVERVIEW	87

1.16.3	FUNCTIONS	88

1.16.4	CRITICAL PATH	89

4	SYNTHESIS	91

5	PERFORMANCE	93

6	OPTIMIZATION	95

FIGURES

Figure 2-1 memory hierarchy of DSP	3

Figure 2-2 top level overview	5

Figure 2-3 DSP top interface	6

Figure 3-1 Ifetch overview	7

Figure 3-2 Ifetch interface	8

Figure 3-3 Reset segment configure	10

Figure 3-4 ITCM structure	11

Figure 3-5 ICache structure	12

Figure 3-6 dispatch overview	15

Figure 3-7 dispatch interface	17

Figure 3-8 Register file in dispatch	26

Figure 3-9 VID allocate and release	27

Figure 3-10 Inexec0 instruction scoreboard	37

Figure 3-11 Intexec1 instruction scoreboard	37

Figure 3-12 Datxfer mem instruction scoreboard	38

Figure 3-13 Datxfer mem1 instruction scoreboard	38

Figure 3-14 Vecexec instruction scoreboard	39

Figure 3-15 intexec0 dispatch group	40

Figure 3-16 intexec1 dispatch group	41

Figure 3-17 datxfer general channel dispatch group	42

Figure 3-18 datxfer LDV channel dispatch group	42

Figure 3-19 vecexec dispatch group	43

Figure 3-20 Queue status suppose 3-dispatch per cycle	50

Figure 3-21 Queue state suppose 2-dispatch per cycle	50

Figure 3-22 integer 0 overview	51

Figure 3-23 integer 1 overview	52

Figure 3-24 integer interface	52

Figure 3-25 ALU0 UNIT structure diagram	53

Figure 3-26 ALU0 UNIT structure diagram	54

Figure 3-27 LOGIC UNIT structure diagram	55

Figure 3-28 SHIFT UNIT structure diagram	56

Figure 3-29 COUNT UNIT structure diagram	57

Figure 3-30 MUL UNIT structure diagram	58

Figure 3-31 clock cycles diagram	59

Figure 3-32 DIV UNIT structure diagram	60

Figure 3-33 datxfer overview	61

Figure 3-34 Datxfer interface	63

Figure 3-35 dcache overview	66

Figure 3-36 dcache interface	67

Figure 3-37	70

Figure 3-38 Vecexec unit overview	70

Figure 3-39	71

Figure 3-40 Vecexec unit interface	71

Figure 3-41	72

Figure 3-42 VECALU UNIT structure diagram	72

Figure 3-43 VECLSU UNIT structure diagram	72

Figure 3-44 VECMUL0 UNIT structure diagram	73

Figure 3-45 VECMUL1 UNIT structure diagram	73

Figure 3-46	74

Figure 3-47 VECSRCH UNIT structure diagram	74

Figure 3-48 VECREORDER UNIT structure diagram	75

Figure 3-49 VECOTHER UNIT structure diagram	76

Figure 3-50 write back overview	77

Figure 3-51 write back unit interface	78

Figure 3-52 timer overview	82

Figure 3-53 intspr overview	83

Figure 3-54 intspr unit interface	84

Figure 3-55 coreout overview	87

TABLES

Table 1 memory parameter	3

Table 2 Decode signal explain	19

Table 3 INT instruction decode	20

Table 4 INT0 instruction decode	22

Table 5 INT1 instruction decode	22

Table 6 MEM instruction decode	23

Table 7 VEC instruction decode	24

Table 8 Register port allocate	26

Table 9 Instruction execution channel	28

Table 10 Integer instruction dispatch truth table	32

Table 11 User mode permission check	48

Table 12 Exceptions detected in dispatch	49

PARAMETERS

Basic frequency	: @500MHZ

Technology		: TSMC 22NM

Power			:

Area			:

HXGPTX ISA

Based on HXGPT ISA.

Support all integer instructions of HXGPT ISA.

Support most of vector instructions of HXGPT ISA.

Support segmentation and hypervisor mode for storage protection.

If you are to review detailed ISA, please refer to ISA document.

FEATURES

- 3-way in-order DSP.

- 4-stage pipeline: FETCH, DISPATCH, EXECUTION, WRITE BACK.

- Branch direction prediction.

- Loop buffer built-in to speed up local loop and save power.

- 4-execution channel: integer0, integer1, LSU, vector.

- Same cycle dispatch for vector LOAD-ALU-STORE

- Separate 256K-ITCM and 512K-DTCM for vector function acceleration.

- Separate 32K-ICACHE and 32K-DCACHE for general function acceleration.

- Built-in FFT unit to accelerate FFT or invert FFT operations.

- Support cache coherency with other MCU in local cluster.

- Support 8/16/32-bit integer, 16/32 fixed point packed SIMD in vector unit.

- Special filter instructions to achieve 100% MAC utilization

- Performance counters.

- Clock gating for all sub units.

SPR DEFINE

Remove TLB/DMA/Kernel related SPRs.

Add segments SPRs:

SEG0	(0x48)		X0000 E000

SEG1	(0X49)		X2000 E000

SEG2	(0X4A)		X4000 E000

SEG3	(0X4B)		X6000 E000

SEG4	(0X4C)		X8000 E000

SEG5	(0X4D)		XC000 E000

SEGC0	(0X4E)		X7070 7070

SEGC1	(0X4F)		X0000 7070

Add TCM related SPRs:

DTCM	(0x46)

ITCM	(0x47)

Add FFT related SPRs:

FFTW	(0x78)		FFT weight

FFTD	(0x79)		FFT data

FFTC	(0x7A)		FFT control word

Extend TIMER related SPRs:

TIMLO/HI	(0x3E/0X3F)

TIMCLO/HI	(0x06/0X07)

All SPRs are initialized as 0s during hardware reset except segment related ones

If you are to review detailed description of SPRs, please refer to ISA document.

OVERVIEW

MEMORY HIERACHY

Figure 2-1 memory hierarchy of DSP

Memory parameters:

Table 1 memory parameter

| General purpose acceleration |  |  |  | Vector purpose acceleration |  |
| --- | --- | --- | --- | --- | --- |
| Memories | Size | Prefetch | Control | Memories | Size |
| ICACHE | 32K/4-WAY | NO | YES | ITCM | 256K |
| DCACHE | 32K/4-WAY | YES | YES | DTCM | 512K |

TCM

- ITCM or DTCM is software-maintained storage.

- The TCM region is defined by ITCM_SPR and DTCM_SPR, the high 16-bit represents compare mask and the low 16-bit represents compare tag.

- TCM is enabled by setting the IT/DT bits in PSC and CPUC. If some address falls into TCM range, fetch data from TCM and it acts as a hit.

- Data TCM is divided into 4 banks to support read and write at the same time.

CACHE

- ICACHE and DCACHE is hardware-maintained storage.

- It is enabled by setting the IC/DC bits in PSC and CPUC. Besides, each segment has its own cacheable bit to control this behavior.

- Cache control instructions are supported to maintain cache: invalid, sync, flush, lock, unlock, and open.

- Prefetch instruction help to fill data cache more quickly. They are supposed to complete when they send request to miss queue.

- Data cache supports coherency with other MCU caches within its cluster, so cache should synchronize its tag and data information with other cores.

PXI

- PXI region is defined by PXITAG_SPR and PXIMASK_SPR.

- DSP can access video memory according to PXI interface. It’s larger than internal data TCM. Each cycle it returns 128-bit data.

- Video memory does not participate in coherency check, so it will return data more quickly than AXI interface.

- PXI accepts data cache requests or instruction cache requests. Besides, cache can also cast out data to PXI.

AXI

- AXI accepts data cache requests or instruction cache requests. Besides, cache can also cast out data to AXI.

- All requests will need to be checked before go to AXI bus. If the other MCU cache has the same cache line, it will send evict to that MCU and wait for its data to cast out.

- AXI interface can return the key word to DSP first and then the whole cache line.

- Both DSP and external bus run at the same frequency. No ASYNC-FIFO is needed.

INTERCONNECT

Figure 2-2 top level overview

- There are two IPs in the cluster: DSP and MCU. They share the same coherency check logic and AXI bus. MCU is light core without vector and large memories.

- The design in red region is DSP. It contains fetch, dispatch, execution units, load/store unit, write back, timer, FFT unit and so on.

- INTSPR is not an independent unit which offers SPR value to write back. Performance counter also works in this unit.

- DATXFER is the load/store unit which supports both integer memory access and vector memory access. Prefetch and cache control are also executed here.

- FFT accelerator is actually a subunit in this design. It takes hundreds of cycles to initial its local memory and execution. When FFT results are ready, SPR read can pull the results out of its internal FIFO.

- Vector unit supports packed SIMD. All vectors computation instructions support 8/16/32-bit data type except MADs which only support 8/16-bit data type.

INTERFACE

If you are to review detailed interface, refer to interface document.

Here is the overview of top interface.

Figure 2-3 DSP top interface

- XWIRE outputs CPU id and version id to INTSPR unit to generate a special SPRA.

- COREOUT unit initial FFT by SPR write. When results are ready, FFT output value to WRITE BACK unit.

- DSP accepts external interrupts by COREIN unit. It’s 10-bit interrupt vector.

- The main interface between DSP and AXI is COREOUT unit which outputs requests to AXISYNC unit to check cache coherency and then throws it out to AXI bus.

- Idle instruction is simple in internal pipeline, just pipe down the opcode and its operation type. COREOUT unit will throw it to EXTAPB to take further steps. DSP memories may also cooperate with idle to sleep or shut down if it’s needed.

- PXI offers an interface for DSP to access a larger local memory which can be regarded as the second level DTCM. The first level DTCM is 512K bytes. PXI interface is never aware of coherency issue and software should take notice of this.

MICRO-ARCHITECTURE

IFETCH

INTRODUCTION

Instruction fetch unit is responsible for fetching instructions from different address space according received fetch request and sends fetched instructions to instruction queue buffer. There will be an instruction permission exception (INSN NOEXEC) if any fetch address error or permission deny occurs. There is also a simple 2-bit branch predictor to deal with control conflict.

OVERVIEW

Figure 3-1 Ifetch overview

INTERFACE

Figure 3-2 Ifetch interface

FUNCTIONS

REQUEST MULTIPLEXER

Fall fetch request comes from dispatch unit. A fall fetch request will be sent when instruction queue not empty (means a reset or a restart or a jump fetch request had been sent before if queue is empty) and free entries are enough for a new line to fill. Jump fetch request comes from branch execution unit. A jump fetch request will be sent when an unconditional jump instruction or a prediction taken branch instruction be detected. Restart fetch request comes from write-back unit. A restart fetch request will be sent when detected branch prediction error or exceptions.

Fall fetch request can’t occur same cycle as jump fetch request because jump request is really detected in dispatch stage so instruction queue had been empty when jump fetch request go to execution stage. Fall and restart, jump and restart requests can be occur in same cycle. The more early instruction caused fetch request, the higher priority. So the priority is restart > jump > fall.

PERMISSION CHECK

There are 6 segments, every segment can be configured by SEGx and SEGxC SPR register. Every segment is composed by more small segments, unit is 216B.

SEGx includes base and size filed, base descripts the base address of the segment, size is used to descript implemented small segment number by bit mask, 0 means implementation.

SEGxC provide 8-bit control for every segments, here only care about two bits: user execution permission (UX) and hypervisor execution permission (HX).

For example, segment 0 base and size can be configured by SEG0 register, and SEG0C[7:0] fields.

- SEG0 high 16-bit base field is “0000” means segment start address is “00000000”

- SEG0 low 16-bit size fields is “FF00” means segments 0 includes 28 small 216B segments

- If UX is 1, user mode can fetch instruction from such segment else generate NO-EXEC permission exception

- If HX is 1, hypervisor mode can fetch instruction from such segment else generate NO-EXEC permission exception

The following is segments configure when reset.

Figure 3-3 Reset segment configure

Every fetch address will be check if the access address is mapped to an implementation small segments, and if the segment execution permission is allowed in current mode. Non-implementation and permission deny both will generate a permission exception and kill fetch request. The permission exception will be sent to dispatch unit.

ADDRESS DIVIDE

Fetch address will be divided to different memory resources if the address don’t generate permission exception. The fetched instruction can come from 3 location: ITCM, ICACHE, and external memory.

ITCM address space can be configured by ITCM SPR register, similar as SEGx SPR. Base field descripts ITCM start address, size field descripts ITCM size. PSC[IT] is used to descript if the ITCM is available.

If the address doesn’t locate at ITCM, we will check if the address is cached or uncacahed. Cache property is descripted in SEGxC[CACHE] field. Cached address will access ICACHE, uncached address will be directly sent to external memory (here the two type access is both sent to icahe_unit).

ITCM LOGIC

ITCM is a small on chip memory, can be quickly access by one cycle. Some memory address space is mapped to ITCM. We can consider it as a software managed instruction cache, instruction fetch can be accelerate by move common used instruction segment to ITCM.

Below is ITCM parameters, it is organized as two 8192*128 single port RAM.

Since ITCM is single port, so there is possible conflict in fetch access and datxfer write access. In such case, write access have higher priority, and fetch request maybe wait one or more cycles. Below is ITCM interface and functional structure.

Figure 3-4 ITCM structure

An old fetch followed by a new fetch is a possible situation for ITCM. Two request will be response one by one if no conflict. The old request will be dropped if old fetch request at the same cycle with datxfer write request and a new fetch request comes next cycle.

ICACHE LOGIC

ICache is responsible for deal with cache and uncached fetch request and do data communication with core-out memory.

Below is ICache parameters, it is organized as 4 tag memory and 4 data memory. Every tag RAM is one 256*20 single port RAM. Every data RAM is four 256*64 single port RAM.

Below is ICache interface and functional structure. All memory is single port, but there are three different access sources: invalid, fetch, refill.

Figure 3-5 ICache structure

Below is all conflict situations three request may be concurrent.

In situation 0, a fetch request and a refill request come at the same cycle. This means the new fetch request is a restart request so the old fetch response (refill) should be dropped. The priority: fetch > refill.

In situation 1, an invalid request and a fetch request come at the same cycle. This situation means a SPRSETR instruction write to ICCTL occur, so the fetch request is a restart fetch. In such case, we deal with invalid operation and delay fetch request: invalid > fetch.

In situation 2, three request come at the same cycle. Concluding as a result, the priority is: invalid > fetch > refill, so we deal with invalid first, delay the fetch request, and drop the refill request.

BP LOGIC

BP (Branch Predictor) is a 2-bit prediction schemes. Such as we fetch a cache line one time, we must provide a line prediction one time – 2-bit * line size.

Below is BP organization parameters. We use a 256 rows x 16-bit wide with write bit mask one port RAM to implement the predictor.

There is BP read and write conflict since we have only one r/w port. Read comes from instruction fetch request, write comes from branch history update request. Delay write request may cause predictor accuracy reduce but delay read request may directly cause pipeline stall, so we choose to delay write request. BP update request will put first into a 4-entry loop queue buffer and update RAM by buffer when read free and drop newest update request when buffer full.

CRITICAL PATHS

None

DISPATCH

INTRODUCTION

Dispatch unit is responsible for buffer fetched instruction line and decode then dispatch them to execution units. Dispatch width is 3-instruction, and GPR, ADR, BTR, CF, FIXCR register are included in dispatch unit. There also has loop back detection logic to avoid unnecessary instruction fetch to save power.

OVERVIEW

Figure 3-6 dispatch overview

- Insn_Q: Fetched instruction line is pushed into instruction queue, then instruction queue provides instructions for dispatch

- Fall req: monitor queue instruction number to send fall fetch request when queue instruction not enough.

- Decode: decode the first 3 instructions in instruction queue to analyze instruction type and operand type

- Register file: instruction read data to prepare instruction operand, and instruction result is write back to register file.

- VID: VID logic is responsible for STV operation ID allocating and releasing management. STV ID is used to ensure later load not operate in non-finished STV address range.

- Dispatch: according to instruction decode information, operand information, VID information to decide instruction target execution unit and provide corresponding control and data.

- Scoreboard: record dispatched instruction information in pipeline to provide forwarding control.

- Fwd logic: according dispatch instruction information and scoreboard history to generate forward control signals.

- Dispatch group: instruction will be package as control and data groups for every execution channel, and send to every execution unit.

- Perm check: some resources only priority mode can be access, we need check current state access permission.

- Exception detect: all front exceptions are detect here and send to write back unit.

- Loop detect: some short back jump instruction target address maybe in queue range. We can detect such case to avoid unnecessary queue flush and re-fetch operations.

INTERFACE

Figure 3-7 dispatch interface

FUNCTIONS

INSTRUCTION QUEUE

Instruction queue is used as instruction buffer from fetch to execution. At most 3-instruction can be popped from queue per cycle, at most a whole cache line-8 instructions-can be pushed to queue.

The queue entry number is set to 14, because when fall request is sent at T0, T1 can receive new instructions at fastest, we need 6 redundancy instructions besides a cache line to keep pipeline active.

Below are some basic operations of queue:

- When instructions are really be dispatched, queue will adjust pop pointer according dispatch instruction number.

- When a jump or prediction taken branch instruction is dispatched and branch target is not in queue range, the queue is cleaned.

- When a PC relative address branch instruction jump back, and back address in queue range, pop pointer will adjust to back entry.

- When new fetched instructions are accepted by queue, the push pointer is adjust according pushing instruction number.

- When datxfer unit sends a dcache miss signal, the pop pointer is restored to the first instruction after missed load instruction.

- When datxfer unit sends a bank collision signal (ST-LD), the pop pointer is restored to the later load instruction.

- When datxfer unit sends an address match signal (STV-LDV), the pop pointer is restored to the later LDV instruction.

FALL REQUEST

A fall fetch request is send to instruction fetch unit when detect current queue instruction number less than 7 but not empty and no fetch request has send.

The fall request will be killed if detect a jump or prediction taken instruction be dispatched at the same cycle with fall request generate.

Branch taken and restart both will clean instruction queue and send fetch request, so in such case instruction is empty, we won’t send any fall request until new instructions come in.

DECODE

We always decode the first 3 instructions in instruction queue to decide if they should be dispatch. Below table is all decode signal we may be used.

Table 2 Decode signal explain

| signal | explain |
| --- | --- |
| IL | an illegal instruction |
| INT | instruction can be dispatched to INTEXEC0 and INTEXEC1 unit |
| INT0 | instruction can only be dispatched to INTEXEC0 unit |
| INT1 | instruction can only be dispatched to INTEXEC1 unit |
| MEM | instruction can be dispatched to DATXFER unit |
| VEC | instruction can be dispatched to VECEXEC unit |
| RSA | instruction reads RSA register |
| RSB | instruction reads RSB register |
| RSC | instruction reads RSC register |
| RTC | instruction writes RTA register |
| ASA | instruction reads ASA register |
| ASB | instruction reads ASB register |
| ASC | instruction reads ASC register |
| ATA | instruction writes ASA register |
| ATC | instruction writes ATA register |
| TSA | instruction reads TSA register |
| TSB | instruction reads TSB register |
| TSD | instruction reads TSC or TSD register |
| TSE | instruction reads TSE register |
| TTB | instruction writes TTB register |
| SSA | instruction reads SSA register |
| STA | instruction writes STA register |
| CFRD | instruction reads CF register |
| CFWR | instruction writes CF register |
| CRRD | instruction reads FIXCR register |
| CRWR | instruction writes FIXCR register |
| FILT | A vector filter instruction |
| VSA | instruction reads VSA register |
| VSB | instruction reads VSB register |
| VSD | instruction reads VSD register |
| VTA | instruction writes VTA register |
| LD | a load instruction |
| ST | a store instruction |
| DB | a double register access instruction |
| VLS | a vector load or store instruction |
| PC | instruction uses PC register |
| COND | a conditional branch instruction |
| UNCD | an unconditional branch instruction |
| CALL | a returned address save branch instruction |
| DJ | a direct jump instruction |
| BAR | a barrier set instruction |
| I05 | instruction uses a 5-bit immediate |
| I10 | instruction uses a 10-bit immediate |
| I10 | instruction uses a 16-bit immediate |
| I120 | instruction uses a 20-bit immediate |
| SH01 | immediate needs shift left 1-bit |
| SH02 | immediate needs shift left 2-bit |
| SH03 | immediate needs shift left 3-bit |
| SH12 | immediate needs shift left 12-bit |
| SH14 | immediate needs shift left 14-bit |
| SH15 | immediate needs shift left 15-bit |
| TS1 | source 1 data select from BTR |
| ALU | instruction executes in ALU subunit |
| MUL | instruction executes in MUL subunit |
| DIV | instruction executes in DIV subunit |
| LGC | instruction executes in LOGIC subunit |
| SHF | instruction executes in SHIFT subunit |
| CNT | instruction executes in COUNT subunit |
| SPR | instruction executes in SPR subunit |
| E2C | instruction executes 2-cycle |

The following list the decode signal of every instruction.

INT

Table 3 INT instruction decode

| TADDTI | TSE |  | TS1 | TTB |  |  |  | SH02 | ALU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TADDPCIL | PC |  |  | TTB |  |  | I16 | SH14 | ALU |
| TADDPCI | PC |  |  | TTB |  |  | I16 | SH02 | ALU |
| TADDTR | RSA | TSD |  | TTB |  |  |  |  | ALU |
| TSETR | RSA |  |  | TTB |  |  |  |  | LGC |
| AADDPCILB | PC |  |  | ATC |  |  | I16 | SH15 | ALU |
| AADDPCIMB | PC |  |  | ATC |  |  | I16 | SH12 | ALU |
| AADDPCI | PC |  |  | ATC |  |  | I16 | SH02 | ALU |
| AADDRI | RSA |  |  | ATC |  |  |  |  | ALU |
| AADDRR | RSA | RSB |  | ATC |  |  |  |  | ALU |
| AANDAI | ASA |  |  | ATC |  |  |  |  | LGC |
| AANDRI | RSA |  |  | ATC |  |  |  |  | LGC |
| AADDAR | RSA | ASB |  | ATC |  |  |  |  | ALU |
| AADDAI | ASA |  |  | ATC |  |  |  |  | ALU |
| AADDUIB |  |  | ASC | ATC |  |  | I16 |  | ALU |
| AADDUIX |  |  | ASC | ATC |  |  | I16 | SH12 | ALU |
| RSETT |  | TSD | TS1 | RTC |  |  |  |  | LGC |
| RADDAI | ASA |  |  | RTC |  |  |  |  | ALU |
| RADDAR | RSA | ASB |  | RTC |  |  |  |  | ALU |
| RSUBAA | ASA | ASB |  | RTC |  |  |  |  | ALU |
| INSUI |  |  | RSC | RTC |  |  | I16 |  | LGC |
| RSETI |  |  |  | RTC |  |  | I16 |  | LGC |
| AND | RSA | RSB |  | RTC |  |  |  |  | LGC |
| OR | RSA | RSB |  | RTC |  |  |  |  | LGC |
| XOR | RSA | RSB |  | RTC |  |  |  |  | LGC |
| CAND | RSA | RSB |  | RTC |  |  |  |  | LGC |
| ADD | RSA | RSB |  | RTC |  |  |  |  | ALU |
| SUBF | RSA | RSB |  | RTC |  |  |  |  | ALU |
| EXTRS | RSA | RSB |  | RTC |  |  |  |  | SHF |
| EXTRU | RSA | RSB |  | RTC |  |  |  |  | SHF |
| ANDI | RSA |  |  | RTC |  |  |  |  | LGC |
| ORI | RSA |  |  | RTC |  |  |  |  | LGC |
| XORI | RSA |  |  | RTC |  |  |  |  | LGC |
| CANDI | RSA |  |  | RTC |  |  |  |  | LGC |
| ADDI | RSA |  |  | RTC |  |  |  |  | ALU |
| SUBFI | RSA |  |  | RTC |  |  |  |  | ALU |
| EXTRSI | RSA |  |  | RTC |  |  | I05 |  | SHF |
| EXTRUI | RSA |  |  | RTC |  |  | I05 |  | SHF |
| ADDS | RSA | RSB |  | RTC |  | CFWR |  |  | ALU |
| SUBFS | RSA | RSB |  | RTC |  | CFWR |  |  | ALU |
| SEL | RSA | RSB |  | RTC | CFRD |  |  |  | ALU |
| SHLS | RSA | RSB |  | RTC |  | CFWR |  |  | SHF |
| SHRW | RSA | RSB |  | RTC |  |  |  |  | SHF |
| SHRAW | RSA | RSB |  | RTC |  |  |  |  | SHF |
| SHLW | RSA | RSB |  | RTC |  |  |  |  | SHF |
| ADDSI | RSA |  |  | RTC |  | CFWR |  |  | ALU |
| SUBFSI | RSA |  |  | RTC |  | CFWR |  |  | ALU |
| SELI | RSA |  |  | RTC | CFRD |  |  |  | ALU |
| SHLSI | RSA |  |  | RTC |  | CFWR | I05 |  | SHF |
| SHRWI | RSA |  |  | RTC |  |  | I05 |  | SHF |
| SHRAWI | RSA |  |  | RTC |  |  | I05 |  | SHF |
| SHLWI | RSA |  |  | RTC |  |  | I05 |  | SHF |
| ADDC | RSA | RSB |  | RTC | CFRD | CFWR |  |  | ALU |
| SUBFB | RSA | RSB |  | RTC | CFRD | CFWR |  |  | ALU |
| RC | RSA | RSB |  | RTC |  | CFWR |  |  | ALU |
| MIN | RSA | RSB |  | RTC |  |  |  |  | LGC |
| MAX | RSA | RSB |  | RTC |  |  |  |  | LGC |
| REV | RSA |  |  | RTC |  |  |  |  | LGC |
| SWAP | RSA |  |  | RTC |  |  |  |  | LGC |
| EXT | RSA |  |  | RTC |  |  |  |  | LGC |
| SAT | RSA |  |  | RTC |  | CFWR |  |  | LGC |
| CFSETI |  |  |  |  |  |  |  |  | ALU |
| LDUTW | ASA | ATA |  | RTC |  |  | E2C | SH02 | ALU |
| LDUSB | ASA | ATA |  | RTC |  |  | E2C |  | ALU |
| LDUSH | ASA | ATA |  | RTC |  |  | E2C |  | ALU |
| LDUB | ASA | ATA |  | RTC |  |  | E2C |  | ALU |
| LDUH | ASA | ATA |  | RTC |  |  | E2C |  | ALU |
| LDUW | ASA | ATA |  | RTC |  |  | E2C | SH02 | ALU |
| LDUL | ASA | ATA |  | RTC |  |  | E2C |  | ALU |
| STUTW | ASA | ATA | TSB |  |  |  |  | SH02 | ALU |
| STUB | ASA | ATA | RSC |  |  |  |  |  | ALU |
| STUH | ASA | ATA | RSC |  |  |  |  |  | ALU |
| STUW | ASA | ATA | RSC |  |  |  |  | SH02 | ALU |
| STUL | ASA | ATA | RSC |  |  |  |  |  | ALU |
| LDUV | ASA | ATA |  |  |  |  | E2C |  | ALU |
| STUV | ASA | ATA |  |  |  |  |  |  | ALU |

INT0

Table 4 INT0 instruction decode

| FCRSETR | RSA |  |  |  |  | CRWR | LGC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RSETSPR |  |  |  | RTC |  | E2C | SPR |
| SPRSETR | RSA |  |  | BAR |  |  | SPR |
| COUNTNZB | RSA | RSB |  | RTC |  |  | CNT |
| COUNTLS | RSA |  |  | RTC |  |  | CNT |
| COUNTLZ | RSA |  |  | RTC |  |  | CNT |
| COUNTSB | RSA |  |  | RTC |  |  | CNT |
| COUNTTZ | RSA |  |  | RTC |  |  | CNT |
| FIXDIV | RSA | RSB |  | RTC | CFWR | CRRD | DIV |
| DIV | RSA | RSB |  | RTC |  |  | DIV |
| REM | RSA | RSB |  | RTC |  |  | DIV |
| RSETV | VSA | RSB |  | RTC |  |  | LGC |

INT1

Table 5 INT1 instruction decode

| NOP |  |  |  |  |  |  |  |  |  | LGC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IDLE |  |  | BAR |  |  |  |  |  |  | ALU |
| RETFI |  |  | BAR |  |  |  |  |  |  | ALU |
| CALLTI | TSE |  | TS1 |  | UNCD | CALL |  | I16 | SH02 | ALU |
| DCALL |  |  |  | PC | UNCD | CALL | DJ | I20 | SH02 | ALU |
| JTI | TSE |  | TS1 |  | UNCD |  |  | I16 | SH02 | ALU |
| DJ |  |  |  | PC | UNCD |  | DJ | I20 | SH02 | ALU |
| RET |  | TSD | TS1 |  | UNCD |  |  |  |  | ALU |
| SWI |  |  | BAR |  |  |  |  |  |  | ALU |
| TRAP |  |  | BAR |  |  |  |  |  |  | ALU |
| JCAZ | ASA |  | TSA |  | COND |  |  |  |  | ALU |
| JCA | ASA | ASB | TSA |  | COND |  |  |  |  | ALU |
| JCAU | ASA | ASB | TSA |  | COND |  |  |  |  | ALU |
| ASETS | SSA |  | ATC |  |  |  |  |  |  | LGC |
| JCWI | RSA |  | TSA |  | COND |  |  |  |  | ALU |
| JCSWI | RSA |  | TSA |  | COND |  |  |  |  | ALU |
| JCUWI | RSA |  | TSA |  | COND |  |  |  |  | ALU |
| JCW | RSA | RSB | TSA |  | COND |  |  |  |  | ALU |
| JCSW | RSA | RSB | TSA |  | COND |  |  |  |  | ALU |
| JCUW | RSA | RSB | TSA |  | COND |  |  |  |  | ALU |
| JDECNZ | RSA | TSD | RTC |  | COND |  |  |  |  | ALU |
| JDECZ | RSA | TSD | RTC |  | COND |  |  |  |  | ALU |
| DJCW | RSA | RSB |  | PC | COND |  | DJ | I10 | SH02 | ALU |
| DJCSW | RSA | RSB |  | PC | COND |  | DJ | I10 | SH02 | ALU |
| DJCUW | RSA | RSB |  | PC | COND |  | DJ | I10 | SH02 | ALU |
| DJDECNZ | RSA |  | RTC | PC | COND |  | DJ |  | SH02 | ALU |
| DJDECZ | RSA |  | RTC | PC | COND |  | DJ |  | SH02 | ALU |
| SSETA | ASA |  | STA |  |  |  |  |  |  | LGC |
| RSETS | SSA |  | RTC |  |  |  |  |  |  | LGC |
| SSETR | RSA |  | STA |  |  |  |  |  |  | LGC |
| MULSAT | RSA | RSB | RTC | CFWR |  |  |  |  | E2C | MUL |
| FIXMUL | RSA | RSB | RTC | CFWR | CRRD |  |  |  | E2C | MUL |
| MUL | RSA | RSB | RTC |  |  |  |  |  | E2C | MUL |

MEM

Table 6 MEM instruction decode

| DCTL | ASA |  | BAR |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PREFETCH | ASA |  |  | LD |  |  |  | E2C |
| LDITW | ASA |  | TTB | LD |  | SH02 |  | E2C |
| LDISB | ASA |  | RTC | LD |  |  |  | E2C |
| LDISH | ASA |  | RTC | LD |  | SH01 |  | E2C |
| LDIB | ASA |  | RTC | LD |  |  |  | E2C |
| LDIH | ASA |  | RTC | LD |  | SH01 |  | E2C |
| LDIW | ASA |  | RTC | LD |  | SH02 |  | E2C |
| LDIL | ASA |  | RTC | LD | DB | SH03 |  | E2C |
| LDIAW | ASA |  | ATC | LD |  | SH02 |  | E2C |
| LDIA | ASA |  | ATC | LD | DB | SH03 |  | E2C |
| STITW | ASA | TSB |  | ST |  | SH02 |  |  |
| STIB | ASA | RSC |  | ST |  |  |  |  |
| STIH | ASA | RSC |  | ST |  | SH01 |  |  |
| STIW | ASA | RSC |  | ST |  | SH02 |  |  |
| STIL | ASA | RSC |  | ST | DB | SH03 |  |  |
| STIAW | ASA | ASC |  | ST |  | SH02 |  |  |
| STIA | ASA | ASC |  | ST | DB | SH03 |  |  |
| LDUTW | ASA |  | TTB | LD |  | SH02 | ALU | E2C |
| LDUSB | ASA |  | RTC | LD |  |  | ALU | E2C |
| LDUSH | ASA |  | RTC | LD |  |  | ALU | E2C |
| LDUB | ASA |  | RTC | LD |  |  | ALU | E2C |
| LDUH | ASA |  | RTC | LD |  |  | ALU | E2C |
| LDUW | ASA |  | RTC | LD |  | SH02 | ALU | E2C |
| LDUL | ASA |  | RTC | LD | DB |  | ALU | E2C |
| STUTW | ASA | TSB |  | ST |  | SH02 | ALU |  |
| STUB | ASA | RSC |  | ST |  |  | ALU |  |
| STUH | ASA | RSC |  | ST |  |  | ALU |  |
| STUW | ASA | RSC |  | ST |  | SH02 | ALU |  |
| STUL | ASA | RSC |  | ST | DB |  | ALU |  |
| LDV | ASA | RSB | VTA | LD | VLS |  |  | E2C |
| STV | ASA | RSB | VSD | ST | VLS |  |  |  |
| LDUV | ASA | RSB | VTA | LD | VLS | ATA | ALU | E2C |
| STUV | ASA | RSB | VSD | ST | VLS | ATA | ALU |  |

VEC

Table 7 VEC instruction decode

| VCMPS | VSA | VSB |  |  | VTA |
| --- | --- | --- | --- | --- | --- |
| VCMPU | VSA | VSB |  |  | VTA |
| VSEL | VSA | VSB | VSD |  | VTA |
| VCOPY | VSA |  |  |  | VTA |
| VUNPACK | VSA |  |  |  | VTA |
| VPACK | VSA |  |  |  | VTA |
| VBRD | RSA |  |  |  | VTA |
| VTAIL | VSA | RSB |  |  | VTA |
| VSETR | RSA | RSB |  |  | VTA |
| VDUPL | VSA | RSB |  |  | VTA |
| VSPREAD | VSA | RSB |  |  | VTA |
| VDILV | VSA | RSB |  |  | VTA |
| VEXT | VSA | RSB |  |  | VTA |
| VEXTB | VSA | RSB |  |  | VTA |
| VAND | VSA | VSB |  |  | VTA |
| VOR | VSA | VSB |  |  | VTA |
| VXOR | VSA | VSB |  |  | VTA |
| VCAND | VSA | VSB |  |  | VTA |
| VADD | VSA | VSB |  |  | VTA |
| VSUB | VSA | VSB |  |  | VTA |
| VDIFF | VSA | VSB |  |  | VTA |
| VSHL | VSA | VSB |  |  | VTA |
| VSHR | VSA | VSB |  |  | VTA |
| VSHRA | VSA | VSB |  |  | VTA |
| VMAX | VSA | VSB |  |  | VTA |
| VMIN | VSA | VSB |  |  | VTA |
| VMUL | VSA | VSB |  |  | VTA |
| V2MUL | VSA | VSB |  |  | VTA |
| V2MULS | VSA | VSB |  |  | VTA |
| VCONCAT | VSA | RSB | VSD |  | VTA |
| VINS | VSA | RSB | VSD |  | VTA |
| VILV | VSA | RSB | VSD |  | VTA |
| VCSUM | VSA | VSB | VSD | E2C | VTA |
| V2CSUM | VSA | VSB | VSD | E2C | VTA |
| FILTC | VSA | RSB | VSD | FILT |  |
| FILTV | VSA | RSB | VSD | FILT | VTA |
| VMAXA | VSA | VSB |  |  | VTA |
| VMINA | VSA | VSB |  |  | VTA |

REGISTER FILE

All register file in dispatch unit:

- GPR: 16 32-bit general purpose registers

- ADR: 16 32-bit address registers

- BTR: 4 32-bit branch target registers, 3 bits address, only implement 0, 1, 4, 7

- SCR: 8 32-bit scratch registers

- CF: 1 1-bit carry register

- FIXCR: 1 2-bit fix pointer control register.

Figure 3-8 Register file in dispatch

Below is register file port allocate table.

Table 8 Register port allocate

VID LOGIC

STV instruction can only access external memory address, if there are later memory data dependent load instruction, we must ensure STV completely finish--data write to memory and cache synchronization finish-- when load access.

We allocate an ID for every STV operation, called VID, and VID has following operations in dispatch unit:

- Allocate a VID for dispatched STV instruction

- Release VID if STV instruction is canceled

- Release VID if corein feedback a AXI access STV finish

- Release VID if coreout feedback a PXI access STV finish

- Stop dispatch STV instruction if no available VID

- Record restart STV/load conflict restarted VID, stop dispatch until VID is released

All VID are available –set valid to 1—when reset, allocation will set valid from 1 to 0, release will set valid from 0 to 1. No available VID when all valid is 0. There are 16 available VID in total, the allocation and release operation as following figure.

Figure 3-9 VID allocate and release

DISPATCH LOGIC

Dispatch logic is responsible for providing control and operands for every execution channel.

Different instruction will be dispatch to different execution unit, instruction execution cycle will also decide following instruction dispatch.

There are 4 execution unit: intexec0_unit, intexec1_unit, datxfer_unit, vecexec_unit. Intexec0_unit, intexec1_unit, vec_exec_unit can receive one corresponding instruction per cycle. datxfer_unit can receive a scalar load or store instruction per cycle, but can receive a STV and a LDV at the same cycle.

Below table is instruction execution channel and execution cycle information.

Table 9 Instruction execution channel

| instruction | intexec0 | intexec1 | datxfer | vecexec | cycle |
| --- | --- | --- | --- | --- | --- |
| NOP |  | intexec1 |  |  | 1 |
| IDLE |  | intexec1 |  |  | 1 |
| REFI |  | intexec1 |  |  | 1 |
| TADDTI | intexec0 | intexec1 |  |  | 1 |
| TADDPCIL | intexec0 | intexec1 |  |  | 1 |
| TADDPCI | intexec0 | intexec1 |  |  | 1 |
| TADDTR | intexec0 | intexec1 |  |  | 1 |
| CALLTI |  | intexec1 |  |  | 1 |
| DCALL |  | intexec1 |  |  | 1 |
| JTI |  | intexec1 |  |  | 1 |
| DJ |  | intexec1 |  |  | 1 |
| RET |  | intexec1 |  |  | 1 |
| SWI |  | intexec1 |  |  | 1 |
| TRAP |  | intexec1 |  |  | 1 |
| REPET |  | Intexec1 |  |  | 1 |
| TSETR | intexec0 | intexec1 |  |  | 1 |
| FCRSETR | intexec0 |  |  |  | 1 |
| JCAZ |  | intexec1 |  |  | 1 |
| JCA |  | intexec1 |  |  | 1 |
| JCAU |  | intexec1 |  |  | 1 |
| AADDPCILB | intexec0 | intexec1 |  |  | 1 |
| AADDPCIMB | intexec0 | intexec1 |  |  | 1 |
| AADDPCI | intexec0 | intexec1 |  |  | 1 |
| AADDRI | intexec0 | intexec1 |  |  | 1 |
| AADDRR | intexec0 | intexec1 |  |  | 1 |
| ASETS |  | intexec1 |  |  | 1 |
| AANDAI | intexec0 | intexec1 |  |  | 1 |
| AANDRI | intexec0 | intexec1 |  |  | 1 |
| AADDAR | intexec0 | intexec1 |  |  | 1 |
| AADDAI | intexec0 | intexec1 |  |  | 1 |
| AADDUIB | intexec0 | intexec1 |  |  | 1 |
| AADDUIX | intexec0 | intexec1 |  |  | 1 |
| FENCE |  |  | datxfer |  | 2 |
| DCTL |  |  | datxfer |  | n |
| PREFETCH |  |  | datxfer |  | 2 |
| JCWI |  | intexec1 |  |  | 1 |
| JCSWI |  | intexec1 |  |  | 1 |
| JCUWI |  | intexec1 |  |  | 1 |
| JCW |  | intexec1 |  |  | 1 |
| JCSW |  | intexec1 |  |  | 1 |
| JCUW |  | intexec1 |  |  | 1 |
| JDECNZ |  | intexec1 |  |  | 1 |
| JDECZ |  | intexec1 |  |  | 1 |
| DJCW |  | intexec1 |  |  | 1 |
| DJCSW |  | intexec1 |  |  | 1 |
| DJCUW |  | intexec1 |  |  | 1 |
| DJDECNZ |  | intexec1 |  |  | 1 |
| DJDECZ |  | intexec1 |  |  | 1 |
| RSETT | intexec0 | intexec1 |  |  | 1 |
| RADDAI | intexec0 | intexec1 |  |  | 1 |
| RADDAR | intexec0 | intexec1 |  |  | 1 |
| RSUBAA | intexec0 | intexec1 |  |  | 1 |
| SSETA |  | intexec1 |  |  | 1 |
| RSETS |  | intexec1 |  |  | 1 |
| SSETR |  | intexec1 |  |  | 1 |
| INSUI | intexec0 | intexec1 |  |  | 1 |
| RSETI | intexec0 | intexec1 |  |  | 1 |
| RSETSPR | intexec0 |  |  |  | 1 |
| SPRSETR | intexec0 |  |  |  | 1 |
| AND | intexec0 | intexec1 |  |  | 1 |
| OR | intexec0 | intexec1 |  |  | 1 |
| XOR | intexec0 | intexec1 |  |  | 1 |
| CAND | intexec0 | intexec1 |  |  | 1 |
| ADD | intexec0 | intexec1 |  |  | 1 |
| SUBF | intexec0 | intexec1 |  |  | 1 |
| EXTRS | intexec0 | intexec1 |  |  | 1 |
| EXTRU | intexec0 | intexec1 |  |  | 1 |
| ANDI | intexec0 | intexec1 |  |  | 1 |
| ORI | intexec0 | intexec1 |  |  | 1 |
| XORI | intexec0 | intexec1 |  |  | 1 |
| CANDI | intexec0 | intexec1 |  |  | 1 |
| ADDI | intexec0 | intexec1 |  |  | 1 |
| SUBFI | intexec0 | intexec1 |  |  | 1 |
| EXTRSI | intexec0 | intexec1 |  |  | 1 |
| EXTRUI | intexec0 | intexec1 |  |  | 1 |
| COUNTNZB | intexec0 |  |  |  | 1 |
| ADDS | intexec0 | intexec1 |  |  | 1 |
| SUBFS | intexec0 | intexec1 |  |  | 1 |
| MULSAT |  | intexec1 |  |  | 2 |
| SEL | intexec0 | intexec1 |  |  | 1 |
| SHLS | intexec0 | intexec1 |  |  | 1 |
| SHRW | intexec0 | intexec1 |  |  | 1 |
| SHRAW | intexec0 | intexec1 |  |  | 1 |
| SHLW | intexec0 | intexec1 |  |  | 1 |
| ADDSI | intexec0 | intexec1 |  |  | 1 |
| SUBFSI | intexec0 | intexec1 |  |  | 1 |
| SELI | intexec0 | intexec1 |  |  | 1 |
| SHLSI | intexec0 | intexec1 |  |  | 1 |
| SHRWI | intexec0 | intexec1 |  |  | 1 |
| SHRAWI | intexec0 | intexec1 |  |  | 1 |
| SHLWI | intexec0 | intexec1 |  |  | 1 |
| ADDC | intexec0 | intexec1 |  |  | 1 |
| SUBFB | intexec0 | intexec1 |  |  | 1 |
| RC | intexec0 | intexec1 |  |  | 1 |
| MIN | intexec0 | intexec1 |  |  | 1 |
| MAX | intexec0 | intexec1 |  |  | 1 |
| COUNTLS | intexec0 |  |  |  | 1 |
| COUNTLZ | intexec0 |  |  |  | 1 |
| COUNTSB | intexec0 |  |  |  | 1 |
| COUNTTZ | intexec0 |  |  |  | 1 |
| REV | intexec0 | intexec1 |  |  | 1 |
| SWAP | intexec0 | intexec1 |  |  | 1 |
| EXT | intexec0 | intexec1 |  |  | 1 |
| SAT | intexec0 | intexec1 |  |  | 1 |
| FIXMUL |  | intexec1 |  |  | 2 |
| MUL |  | intexec1 |  |  | 2 |
| FIXDIV | intexec0 |  |  |  | n |
| DIV | intexec0 |  |  |  | n |
| REM | intexec0 |  |  |  | n |
| LDITW |  |  | datxfer |  | 2 |
| LDISB |  |  | datxfer |  | 2 |
| LDISH |  |  | datxfer |  | 2 |
| LDIB |  |  | datxfer |  | 2 |
| LDIH |  |  | datxfer |  | 2 |
| LDIW |  |  | datxfer |  | 2 |
| LDIL |  |  | datxfer |  | 2 |
| LDIAW |  |  | datxfer |  | 2 |
| LDIA |  |  | datxfer |  | 2 |
| STITW |  |  | datxfer |  | 1 |
| STIB |  |  | datxfer |  | 1 |
| STIH |  |  | datxfer |  | 1 |
| STIW |  |  | datxfer |  | 1 |
| STIL |  |  | datxfer |  | 1 |
| STIAW |  |  | datxfer |  | 1 |
| STIA |  |  | datxfer |  | 1 |
| LDUTW | intexec0 | intexec1 | datxfer |  | 2 |
| LDUSB | intexec0 | intexec1 | datxfer |  | 2 |
| LDUSH | intexec0 | intexec1 | datxfer |  | 2 |
| LDUB | intexec0 | intexec1 | datxfer |  | 2 |
| LDUH | intexec0 | intexec1 | datxfer |  | 2 |
| LDUW | intexec0 | intexec1 | datxfer |  | 2 |
| LDUL | intexec0 | intexec1 | datxfer |  | 2 |
| STUTW | intexec0 | intexec1 | datxfer |  | 1 |
| STUB | intexec0 | intexec1 | datxfer |  | 1 |
| STUH | intexec0 | intexec1 | datxfer |  | 1 |
| STUW | intexec0 | intexec1 | datxfer |  | 1 |
| STUL | intexec0 | intexec1 | datxfer |  | 1 |
| RSETV | intexec0 |  |  |  | 1 |
| LDV |  |  | datxfer |  | 2 |
| STV |  |  | datxfer |  | 1 |
| LDUV | intexec0 | intexec1 | datxfer |  | 2 |
| STUV | intexec0 | intexec1 | datxfer |  | 1 |
| VCMPS |  |  |  | vecexec | 1 |
| VCMPU |  |  |  | vecexec | 1 |
| VSEL |  |  |  | vecexec | 1 |
| VCOPY |  |  |  | vecexec | 1 |
| VUNPACK |  |  |  | vecexec | 1 |
| VPACK |  |  |  | vecexec | 1 |
| VBRD |  |  |  | vecexec | 1 |
| VTAIL |  |  |  | vecexec | 1 |
| VSETR |  |  |  | vecexec | 1 |
| VDUPL |  |  |  | vecexec | 1 |
| VSPREAD |  |  |  | vecexec | 1 |
| VDILV |  |  |  | vecexec | 1 |
| VEXT |  |  |  | vecexec | 1 |
| VEXTB |  |  |  | vecexec | 1 |
| VAND |  |  |  | vecexec | 1 |
| VOR |  |  |  | vecexec | 1 |
| VXOR |  |  |  | vecexec | 1 |
| VCAND |  |  |  | vecexec | 1 |
| VADD |  |  |  | vecexec | 1 |
| VSUB |  |  |  | vecexec | 1 |
| VDIFF |  |  |  | vecexec | 1 |
| VSHL |  |  |  | vecexec | 1 |
| VSHR |  |  |  | vecexec | 1 |
| VSHRA |  |  |  | vecexec | 1 |
| VMAX |  |  |  | vecexec | 1 |
| VMIN |  |  |  | vecexec | 1 |
| VMUL |  |  |  | vecexec | 1 |
| V2MUL |  |  |  | vecexec | 1 |
| V2MULS |  |  |  | vecexec | 1 |
| VCONCAT |  |  |  | vecexec | 1 |
| VINS |  |  |  | vecexec | 1 |
| VILV |  |  |  | vecexec | 1 |
| VCSUM |  |  |  | vecexec | 2 |
| V2CSUM |  |  |  | vecexec | 2 |
| VFILTC |  |  |  | vecexec | n |
| VFILTV |  |  |  | vecexec | n |
| VMAXA |  |  |  | vecexec | 1 |
| VMINA |  |  |  | vecexec | 1 |

There are 5-type dispatch logic to consider:

- Vecexec instruction: always select the first valid vecexec instruction in the 3 dispatch instructions.

- Non-update datxfer instruction: always select the first valid datxfer instruction in the 3-dispatch instructions.

- Update datxfer instruction: the first valid update instruction can be dispatch when there is neither datxfer unit conflict nor integer unit conflict.

- LDV instruction: if STV is the first datxfer instruction and LDV is the second datxfer instruction, the two instruction can both be dispatched.

- Integer instruction: always can dispatch 2 integer instruction if not the first integer and the second integer instruction are both INT0 or both INT1.

The integer instruction dispatch logic is more complex, below table is all case of integer instruction combination (0 in first 3 column means not integer instruction, X in last 2 column means no valid integer instruction can be dispatch from that channel, 0/1/2 in last 2 column means instruction slot number).

Table 10 Integer instruction dispatch truth table

| insn0 | insn1 | insn2 | INT0 | INT1 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | X | X |
| 0 | 0 | INT | 2 | X |
| 0 | 0 | INT0 | 2 | X |
| 0 | 0 | INT1 | X | 2 |
| 0 | INT | 0 | 1 | X |
| 0 | INT | INT | 1 | 2 |
| 0 | INT | INT0 | 2 | 1 |
| 0 | INT | INT1 | 1 | 2 |
| 0 | INT0 | 0 | 1 | X |
| 0 | INT0 | INT | 1 | 2 |
| 0 | INT0 | INT0 | 1 | X |
| 0 | INT0 | INT1 | 1 | 2 |
| 0 | INT1 | 0 | X | 1 |
| 0 | INT1 | INT | 2 | 1 |
| 0 | INT1 | INT0 | 2 | 1 |
| 0 | INT1 | INT1 | X | 1 |
| INT | 0 | 0 | 0 | X |
| INT | 0 | INT | 0 | 2 |
| INT | 0 | INT0 | 2 | 0 |
| INT | 0 | INT1 | 0 | 2 |
| INT | INT | 0 | 0 | 1 |
| INT | INT | INT | 0 | 1 |
| INT | INT | INT0 | 0 | 1 |
| INT | INT | INT1 | 0 | 1 |
| INT | INT0 | 0 | 1 | 0 |
| INT | INT0 | INT | 1 | 0 |
| INT | INT0 | INT0 | 1 | 0 |
| INT | INT0 | INT1 | 1 | 0 |
| INT | INT1 | 0 | 0 | 1 |
| INT | INT1 | INT | 0 | 1 |
| INT | INT1 | INT0 | 0 | 1 |
| INT | INT1 | INT1 | 0 | 1 |
| INT0 | 0 | 0 | 0 | X |
| INT0 | 0 | INT | 0 | 2 |
| INT0 | 0 | INT0 | 0 | X |
| INT0 | 0 | INT1 | 0 | 2 |
| INT0 | INT | 0 | 0 | 1 |
| INT0 | INT | INT | 0 | 1 |
| INT0 | INT | INT0 | 0 | 1 |
| INT0 | INT | INT1 | 0 | 1 |
| INT0 | INT0 | 0 | 0 | X |
| INT0 | INT0 | INT | 0 | X |
| INT0 | INT0 | INT0 | 0 | X |
| INT0 | INT0 | INT1 | 0 | X |
| INT0 | INT1 | 0 | 0 | 1 |
| INT0 | INT1 | INT | 0 | 1 |
| INT0 | INT1 | INT0 | 0 | 1 |
| INT0 | INT1 | INT1 | 0 | 1 |
| INT1 | 0 | 0 | X | 0 |
| INT1 | 0 | INT | 2 | 0 |
| INT1 | 0 | INT0 | 2 | 0 |
| INT1 | 0 | INT1 | X | 0 |
| INT1 | INT | 0 | 1 | 0 |
| INT1 | INT | INT | 1 | 0 |
| INT1 | INT | INT0 | 1 | 0 |
| INT1 | INT | INT1 | 1 | 0 |
| INT1 | INT0 | 0 | 1 | 0 |
| INT1 | INT0 | INT | 1 | 0 |
| INT1 | INT0 | INT0 | 1 | 0 |
| INT1 | INT0 | INT1 | 1 | 0 |
| INT1 | INT1 | 0 | X | 0 |
| INT1 | INT1 | INT | X | 0 |
| INT1 | INT1 | INT0 | X | 0 |
| INT1 | INT1 | INT1 | X | 0 |

We can get below logic from above table:

Update instruction will generate an INT instruction and a datxfer instruction, so must two sub instructions have no conflict the update instruction can be dispatch.

Some other cases will also decide if instruction can really be dispatched. We divide them to 4 class: stall cases, wait cases, depend cases, stop cases.

Stall cases are situation that will stop all following instruction dispatch, as following:

- Reset

- Restart

- Barrier is set

- DIV is executing

- DCTL is executing

- Bank collision or address match occur

- Waiting a VID release

- Miss occur until miss deal done

- Miss queue full

- Corein not available

Wait cases are situation special instruction operand not available or some other resources not available that we must let instruction wait, as following:

- Dispatch a FIXCR use instruction but a FIXCR write instruction not finish

- Dispatch an instruction the source register is same as an executing double load high destination register

- Dispatch an 1-cycle instruction the source register is same as last cycle load instruction destination register

- Dispatch an 1-cycle instruction the source register is same as last cycle MUL instruction destination register

- Dispatch an 1-cycle instruction the source register is same as last cycle RSETSPR instruction destination register

- Dispatch an 1-cycle INTEXEC1 instruction if last cycle is a 2-cyle MUL instruction

- Dispatch an 1-cycle VECEXEC instruction if last cycle is a 2-cycle VECEXEC instruction

- Dispatch an STV instruction but no usable VID

Depend cases are situation same cycle instructions have data dependence or resources conflict, as following:

- Behind instruction source register is same as front instruction destination register

- RSETV and VEXEXEC instruction vector register read port conflict

Stop cases are situation stop following instruction dispatch in same cycle, as following:

- A branch taken jump instruction

- The load instruction followed by not 2-cycle instruction or DIV instruction

- The MUL instruction followed by not 2-cycle instruction or DIV instruction

- A DIV instruction is dispatching

- A barrier-set instruction is dispatching

- An instruction followed by illegal or permission deny instruction

SCOREBOARD

Scoreboard records all dispatched instruction destination register address and write enable in pipeline in order to generate forward control signals for later dispatched instruction.

1-cycle instruction pipeline is: IF – ID – EX –WB; 2-cycle instruction pipeline IF – ID –EX0 – EX1 – WB.

2-cycle instruction will first record in WAIT if they go EX0 stage, because in such stage following instruction must wait if they will use EX0 instruction result as the results not produced. Wait record will transfer to EX then transfer to WB one cycle by one cycle.

1-cycle instruction will first record in EX if they are dispatched to EX stage. EX record will be transfer to WB after one cycle.

WAIT record means instruction results not available in current cycle so data dependent instruction must wait. EX and WB record means results available in current cycle so can provide data forward for data dependent instruction.

INTEXEC0

Below is INTEXEC0 instruction scoreboard logic.

Figure 3-10 Inexec0 instruction scoreboard

- Set EX address and write enable from insn_int0 when INT0_INSN is dispatching

- Keep EX address and write enable as DIV instruction when DIV instruction is executing

- Clear EX any other case

- Set WB address and write enable from EX when no stall

- Clear WB any other case

INTEXEC1

In INTEXEC1 channel, only MUL instruction is 2 execution cycle, all others are 1 execution cycle. Below is INTEXEC1 instruction scoreboard logic.

Figure 3-11 Intexec1 instruction scoreboard

- Set WAIT address and write enable from insn_int1 when an MUL is dispatching

- Clear WAIT any other case

- Set EX address and write enable from insn_int1 when an non-MUL is dispatching

- Set EX address and write enable from WAIT when no instruction is dispatching and no stall

- Clear EX any other case

- Set WB address and write enable from EX when no stall

- Clear WB any other case

DATXFER

In DATXFER channel, all load instructions are 2 execution cycle if no dcache access miss. Datxfer channel has two sub-channel, MEM and MEM1. MEM1 can only pass LDV instruction. Below is their scoreboard logic.

Figure 3-12 Datxfer mem instruction scoreboard

- Set WAIT address and write enable from insn_mem when an MEM instruction is dispatching

- Clear WAIT any other case

- Set EX address and write enable from WAIT when no stall

- Set EX address and write enable from WB when datxfer send a dcache miss signal

- Clear EX any other case

- Set WB address and write enable from EX when no stall

- Clear WB any other case

Figure 3-13 Datxfer mem1 instruction scoreboard

- Set WAIT address and write enable from insn_mem when anMEM instruction is dispatching

- Clear WAIT any other case

- Set EX address and write enable from WAIT when no stall

- Clear EX any other case

VECEXEC

No need to restore LDV from WB to EX record because vector register is read in EX stage, and when the dependent data use vector instruction pipeline to EX stage, the miss LDV instruction have write back data to register file.

In VECEXEC channel, filter instruction is n execution cycles,  accumulate instruction is 2 execution cycles, all others are 1 execution cycle. Below is VECEXEC instruction scoreboard logic.

Figure 3-14 Vecexec instruction scoreboard

- Set WAIT address and write enable from insn_vec when a 2 cycle vector instruction is dispatching

- Clear WAIT any other case

- Set EX address and write enable from insn_vec when an 1 cycle vector instruction is dispatching

- Set EX address and write enable from WAIT when no instruction is dispatching and no stall

- Keep EX address and write enable as filter instruction when filter instruction is executing

- Clear EX any other case

- Set WB address and write enable from EX when no stall

- Clear WB any other case

DISPATCH GROUP

Dispatch unit will package control signals and operands for every execution unit. Control include instruction opcode information and forward control signals for every source operand. Operands include all source register value or address (for vector register read), destination register address and write enable.

Forward control will be introduction in next section – forward control. Now we look every group data package for every execution unit.

I list all field name that may be used by every channel instructions. Different instruction use different register address fields to access register file, and the operands is put in different source position. All mux controls are come from corresponding decode signals. For example, RSA/RSC-R0 means that we use RSC as R0 read port access address if RSC of instruction decode is set, else use RSA.

INTEXEC0

Below is intexec0 operands overview.

Figure 3-15 intexec0 dispatch group

- R0: GPR 0 read port access address comes from RSC if decode_RSC is valid, ELSE from RSA

- R1: GPR 1 read port access address comes from RSB

- A0: ADR 0 read port access address comes from ASC if decode_ASC is valid, else from ASA

- A1: ADR 1 read port access address comes from ASB

- T0: BTR 0 read port access address comes from TSE if decode_TSE is valid, else from TSD

- VSA_addr: insn_int0[15:12]

- SRC1_val: PC if decode_PC is valid; else T0 if decode_TS1 is valid; else A0 if decode_ASA or decode_ASC is valid; else R0

- SRC2_val: T0 if TADDTR instruction; else A1 if decode_ASB is valid; else R1 if decode_RSB is valid; else 16 if decode_VLS is valid; else immediate.

- SRC3_val: FIXCR . CF if decode_CFRD or decode_CRRD is valid; else immediate

- DST_en: decode_CRWR . decode_CFWR . decode_TTB . decode_ATC or decode_ATA . decode_RTC

- DST_addr: ATA  if decode_ATA is valid; else RTC

INTEXEC1

Below is intexec1 operands overview.

Figure 3-16 intexec1 dispatch group

- R2: GPR 2 read port access address comes from RSC if decode_RSC is valid, ELSE from RSA

- R3: GPR 3 read port access address comes from RSB

- A2: ADR 2 read port access address comes from ASC if decode_ASC is valid, else from ASA

- A3: ADR 3 read port access address comes from ASB

- T1: BTR 1 read port access address comes from TSA if decode_TSA is valid; else from TSE if decode_TSE is valid, else from TSD

- S0: SCR only read port access address from SSA

- SRC1_val: S0 if decode_SSA is valid; else PC if decode_PC is valid; else T1 if decode_TS1 is valid; else A2 if decode_ASA or decode_ASC is valid; else R2

- SRC2_val: T1 if TADDTR instruction; else A3 if decode_ASB is valid; else R3 if decode_RSB is valid; else 16 if decode_VLS is valid; else 0 if RET instruction; else immediate.

- SRC3_val: insn_int1[1] . FIXCR . CF if decode_CFRD or decode_CRRD is valid; else bp_index if decode_COND is valid and decode_DJ is invalid; else immediate

- DST_en: decode_STA . decode_CFWR . decode_TTB . decode_ATC or decode_ATA . decode_RTC

- DST_addr: ATA if decode_ATA is valid; else RTC

DATXFER

Below is intexec1 operands overview.

Figure 3-17 datxfer general channel dispatch group

- R4: GPR 4 read port access address comes from RSC

- R5: GPR 5 read port access address comes from RSC+1

- A4: ADR 4 read port access address comes from ASA

- A5: ADR 5 read port access address comes from ASC

- A6: ADR 6 read port access address comes from ASC+1

- VSD_addr: insn_mem[19:16]

- SRC1_val: A4

- SRC2_val: T2 if decode_TSB is valid; else A5 if decode_ASC is valid; else R4

- SRC3_val: A6 if decode_ASC is valid; else R6

- DST_en: decode_TTB . decode_ATC . decode_RTC

- DST_addr: RTC

Figure 3-18 datxfer LDV channel dispatch group

- A5: ADR 5 read port access address comes from ASA

- SRC1_val: A5

- DST_addr: VTA

VECEXEC

Figure 3-19 vecexec dispatch group

- R6: GPR 6 read port access address comes from RSA

- R7: GPR 7 read port access address comes from RSB

- VSA_addr: insn_vec[15:12]

- SRC1_val: R6 if decode_RSA is valid; else C2 low 32-bit

- VSB_addr: insn_vec[11:8]

- SRC3_val: R5 if decode_RSB is valid; else C2 high 8- bit

- MSA_addr: insn_vec[5:4]

- SRC4_val: immediate

- DST_en: decode_CTA . decode_VTA . decode_MTA

- DST_addr: VTA

SPECIAL NOTE

- A5 read port is used together by insn_mem and insn_mem1, used by insn_mem1 if mem1 instruction is valid; else used by insn_mem

- RSETV read VEC register: need transfer VSA address and RSB value to VECEXEC. RSETV and other vector instruction can’t be dispatched together, so we merge RSET VSA address and RSB value transfer channel with vector instruction VSA address and RSB value channel.

FORWARD LOGIC

For all source operands from register, we need check scoreboard to decide if the register value is the newest. If we detected some instructions are producing a new value for some register, and the results are available when the instruction really use the data, we can forward the newest register value for consumer instructions.

If the instruction used register type and address both same as scoreboard record, it means a register match, we need provide a valid forward signal.

INTEXEC0

There are 3 source operand need forward signals. And source 1 forward divide into integer register forward and vector register forward.

The source 1 enable and address logic as following:

The source 1 forward signals and means as following:

The source 2 enable and address logic as following:

The source 2 forward signals and means as following:

The source 3 enable and address logic as following:

The source 3 forward signals and means as following:

INTEXEC1

The source 1 enable and address logic as following:

The source 1 forward signals and means as following:

The source 2 enable and address logic as following:

The source 2 forward signals and means as following:

The source 3 enable and address logic as following:

The source 3 forward signals and means as following:

The source 4 enable and address logic as following:

The source 4 forward signals and means as following:

DATXFER

The source 1 enable and address logic as following:

The source 1 forward signals and means as following:

The source 2 enable and address logic as following:

The source 2 forward signals and means as following:

The source 3 enable and address logic as following:

The source 3 forward signals and means as following:

The source 1 enable and address logic as following:

The source 1 forward signals and means as following:

VECEXEC

The source 1 enable and address logic as following:

The source 1 forward signals and means as following:

The source 2 enable and address logic as following:

The source 2 forward signals and means as following:

The source 3 enable and address logic as following:

The source 3 forward signals and means as following:

The source 4 enable and address logic as following:

The source 4 forward signals and means as following:

PERMISSION CHECK

User mode have no some priority instruction execution permission, can’t access SCR register file, and can’t access some ADR registers.

Below table lists user mode permission check cases.

Table 11 User mode permission check

| access resources |  | user mode permission |
| --- | --- | --- |
| IDLE | execute an IDLE instruction |  |
| RETFI | execute a RETFI instruction |  |
| SWI | execute a SWI instruction |  |
| TRAP | execute a TRAP instruction |  |
| DCTL | execute a DCTL instruction |  |
| SPRSETR | execute a SPRSETR instruction |  |
| SCR | access scratch registers |  |
| ADR[10:0] | access address registers from 0 to 10 |  |
| ADR[15:11] | access address registers from 11 to 15 |  |

EXCEPTION DETECT

There are 5 exceptions be detected in dispatch unit.

Table 12 Exceptions detected in dispatch

| Exception name | Description |
| --- | --- |
| UNAVAIL_VEC | vector unit not available |
| INSN_BAD | undefined instruction |
| INSN_NOEXEC | Fetch address have no execution permission |
| INSN_PERM | attempted to execute an instruction without appropriate permission |
| DBG_INSN | debug instruction match |

- An UNAVAIL_VEC exception is generated if a vector instruction is dispatching but vector disable control signal is set (PSC[VD] ).

- An INSN_BAD exception is generated if detected an illegal instruction is dispatching. Insn_bad exception can only be sent in slot 0.

- An INSN_NOEXEC is the permission information of instruction fetch from ifetch unit.

- An INSN_PERM exception is generated by permission check logic in dispatch. Insn_perm exception can only be sent in slot 0.

- A DBG_INSN exception is generated if instruction match enable is valid, and instructions address match IM1E/IM0M/IM0C or IM1E/IM1M/IM1C address.

LOOP DETECT

We generally flush instruction queue and send a refill fetch request to fetch unit when an unconditional jump or a prediction taken branch instruction is dispatched. But the jump target instruction maybe still stay in instruction queue if it is a short back jump.

So the loop detect logic is used to detect short back jump target in queue instruction range to avoid queue instruction flush and re-fetch in order to save power and improve dispatch performance.

Figure 3-20 Queue status suppose 3-dispatch per cycle

Figure 3-21 Queue state suppose 2-dispatch per cycle

Above is queue state of we can used back space. The first line number is remain instruction number. The second line number is back space.

REPEAT LOGIC

When detect a REPEAT instruction, the following instruction sequence will be dispatch n times.

CRITICAL PATHS

None

INTEGER

INTRODUCTION

INTEGER unit is responsible for address/target generation, branch judgement and all integer instructions: arithmetic, logic, count, shift, multiplication and division.

Most of the integer instructions are single cycle latency except multiplication (2 cycles) and division (multiple cycles). Integer operations are divided into two groups to be executed on integer 0 and integer 1. If you are to confirm which unit the integer instruction belongs to, please refer to micro-arch design sheet.

OVERVIEW

Figure 3-22 integer 0 overview

- All forwarding results come from back-end latches to be friendly to layout.

- Integer 0 only check SPR index and generate exceptions if violation is found. Read happens in INTSPR unit.

Figure 3-23 integer 1 overview

- Compared to integer 0, integer 1 supports all branch instructions. Branch judgement logic is in LOGIC unit and its new target is generated in ALU.

INTERFACE

Here is the overview of integer interface.

Figure 3-24 integer interface

FUNCTIONS

ALU0 UNIT

ALU0 UNIT support the addition and subtraction type of instruction in arithmetic operations. Below is the structure diagram of ALU0.

Figure 3-25 ALU0 UNIT structure diagram

In addition to normal addition and subtraction operations, ALU performs some special manipulation of the source operations or results. There are several types:

- The one of source operations of TADDTR, AADDRR, AADDAR and RADDAR instructions should shift left before calculating.

- The result of RSUBAA should shift right after calculating.

- The instruction SEL only selects instruction according to carry bit.

- The calculation may overflow, so there is a saturation checking.

- In addition to the result of calculation, one output is the CF bit which is the same as overflow bit.

ALU1 UNIT

Comparing to ALU0, ALU1 supports Branch and Jump instruction. Below is the structure diagram of ALU1.

Figure 3-26 ALU0 UNIT structure diagram

ALU1 can support whatever ALU0 supports, so no need to repeat it here. Here is the description of the instructions which only belongs to ALU1.

- JDECNZ, JDECCZ, DJDECNZ and DJDECZ completes with a source minus 1 to target register, so the output value has one more option.

- Branch instructions taken or Jump instructions execution changes the pc, so the output has a new pc.

- The Branch instructions need to update the prediction information after execution, so taken is out.

LOGIC UNIT

LOGIN UNIT supports most logic operation, for example: AND, OR, ANDI… In addition to those logic operations, LOGIC UNIT also support some other instructions, which is descripted below. Here is the structure diagram of LOGIC UNIT.

Figure 3-27 LOGIC UNIT structure diagram

Descriptions of some specific instructions:

- LOGIC UNIT supports some register move instructions, including general purpose registers, address registers, target registers, special purpose registers. Those instructions include RSETT, RSETS, SSETR, SSETA…

- LOGIC UNIT supports instructions REV and SWAP.

- LOGIC UNIT supports extend instructions.

- LOGIC UNIT supports some saturation instructions.

- LOGIC UNIT supports instructions MAX and MIN.

- LOGIC UNIT supports instruction INSUI.

- In addition, LOGIC UNIT has an operand comparison output for ALU1.

SHIFT UNIT

SHIFT UNIT supports all shift instructions, including shift left, shift right, and shift right arithmetic. In addition to shifting, sometimes the results need to be processed, for example: saturating and extract. Below is the structure diagram of SHIFT UNIT.

Figure 3-28 SHIFT UNIT structure diagram

There are three types of shift instructions.

The first one is normal shift instructions including logic shift and arithmetic shift.

The second one saturates the results of the shift.

The third one will pick out several bits after shifting and sign-extend it.

COUNT UNIT

COUNT UNIT supports COUNT type instructions, including COUNTNZB, COUNTLS, COUNTLZ, COUNTSB, COUNTTZ. Those instructions all calculate the numbers of 0 or 1 of data. Below is the structure diagram of COUNT UNIT.

Figure 3-29 COUNT UNIT structure diagram

MUL UNIT

MUL UNIT supports MUL, MULSAT, FIXMUL. Instruction MULSAT means saturating the result after multiplying. And FIXMUL is fixed point multiply, of which the width is 16 or 32. It takes two cycles for multiplication. There is no doubt that multiplication occupies the largest area in integer. Below is the structure diagram of MUL UNIT.

Figure 3-30 MUL UNIT structure diagram

DIVIDE UNIT

DIVIDE UNIT supports instructions DIV, FIXDIV, REM. It takes multiple cycles to finish division. Each cycle, two-bit quotient is generated. If exception is found in the first cycle, there is no extra hardware to complete the state machine earlier. Here is the structure diagram of DIVIDE UNIT.

Figure 3-31 clock cycles diagram

Figure 3-32 DIV UNIT structure diagram

CRITICAL PATHS

- Result mux is traditional critical path.

- New refill request to IFETCH unit is also critical path

- 32-bit fixed point multiplication execution is critical

DATXFER

INTRODUCTION

DATXFER unit is responsible for data transfer between memories and registers. It contains integer load, integer store, vector load, vector store, prefetch and cache control command. To speed up save and recovery process of function calls, integer memory access supports double word which is 64-bit width. Vector only supports at most 128-bit width, if the address is not aligned, load or store at 128-bit boundary. DATXFER unit can accept one integer operation or one vector load-store couple each cycle. If you are to confirm which instruction belongs to DATXFER unit, please refer to micro-arch design.

OVERVIEW

Figure 3-33 datxfer overview

- Software can define at most 6 segments, each segment has its own permission control bits: no-sync, hyper-exe, hyper-read, hyper-write, user-exe, user-read, user-write, cache. Send logic and exception logic will care about these properties.

- STV CAM is a structure to ensure the right order between external STV and following integer memory operations. If their addresses are matched, restart exception is set.

- Load writes back result in the second execution cycle, so the information should be latched in local.

- Cache has only one port for tag and data to save area. Load and store access data in different cycle which will cause data port collision. This will also occur in TCM access case. Collision logic will check the banks between last-cycle store and current load, if their bank match, recover dispatch and dispatch the load in next cycle.

- Vector load and vector store can be dispatched at the same cycle. In this case, store must come first. The following load may have address dependency with store which will also be detected by collision logic and cause recovery.

- Both load and store support mask operation, which means that load can choose bytes to write to register and store can choose bytes to write to memory.

INTERFACE

Here is the overview of datxfer interface.

Figure 3-34 Datxfer interface

FUNCTIONS

DCTL LOGIC

DCTL is a kind of instruction with alterable latency. Pending miss and insufficient cast-out queue will block dctl. When dctl is sent to cache successfully, dispatch new instructions again. Pending lock will block all following memory access.

SEGMENTS

Each segment has a SPR to define the range. Low 16 bits represents the mask and high 16 bits represents the tag. The match condition is: address (31-16) & mask == tag;

During reset, the segments are initialized and occupy 32-bit address space:

| Tag | Mask | No-sync | hyper permission | user-permission | cache |
| --- | --- | --- | --- | --- | --- |
| 0X0000 | 0XE000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |
| 0X2000 | 0XE000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |
| 0X4000 | 0XE000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |
| 0X6000 | 0XE000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |
| 0X8000 | 0XC000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |
| 0XC000 | 0XC000 | 0 | (E)1(R)1(W)1 | (E)0(R)0(W)0 | 0 |

Software can set the region of each segment and the segments can be discontinuous. If some address falls out of all segments, it will generate address undefined exception.

SEND LOGIC

Load send condition:

- Valid load instruction from dispatch without load miss in current cycle.

- No load exceptions.

- No bank collision with last cycle store.

- According to ITCM/DTCM SPR check result to decide which memory to go.

Store send condition:

- Valid store instruction from dispatch without load miss in current cycle.

- No load/store exceptions.

- According to ITCM/DTCM SPR check result to decide which memory to go.

DCTL send condition:

- Valid DCTL instruction from dispatch without load miss in current cycle.

- Cast-out queue is available.

- No cache miss request pending.

- No load/DCTL exceptions.

COLLISION CHECK

Last cycle store records the following information: cache write, vector, bank.

Collision cases (bank = address (4-3)):

- None vector load writing to bank 3 and the store writes to cache.

- None vector load write to the same bank with none vector store.

- At least one of them is vector and they writes to the same high/low 128 bit.

LOAD MISS

Here is the pipeline for load miss

T0		T1		T2		T3

Load (miss)	DISP	RD		EXT		WB

ALU0				DISP	EXE		WB

ALU1						DISP	EXE

ALU2								DISP

The first load miss occurs which means the following instructions should re-execute. Considering the earliest cycle to output miss is in T3, DISPATCH unit will hold ALU2 in instruction queue. ALU0 and ALU1 will re-execute which will need DISPATCH unit to recover and clear EXE/WB stage. The control logic within DATXFER unit is also quite simple. All load information will pipe to WB stage naturally when valid load comes which will ensure that the information will stay unchanged. If load miss occurs, kill load write back enable in WB stage. Assert enable and refresh result when the data is back.

CRITICAL PATHS

- Memory access operations to cache is critical path.

- Results write back is critical path.

DCACHE

INTRODUCTION

DCACHE unit is responsible for data cache access and maintaining. At the end of EXE stage, DATXFER unit sends out memory access request. In EXE2 stage, read tag out of memory and compare it with address tag to see whether the address is hit in cache. If cache hits, return the data immediately and continue data extension in DATXFER unit. If cache misses, push the request into miss queue and fetch the data from external. When data returns from external, refill cache and cast out cache line if it is necessary. DCTL updates tag memory and cast out cache line if it’s FLUSH or SYNC. When cache hits, prefetch acts as NOP, or it’s pushed into miss queue without blocking pipeline.

OVERVIEW

Figure 3-35 dcache overview

- Tag memory is 20 bits width containing valid bit.

- Data memory is divided into 4 banks. The last bank contains dirty bit.

- It’s 4 way set cache. Each cache line is 32 bytes width and there are 256 lines in it.

INTERFACE

Here is the overview of dcache interface.

Figure 3-36 dcache interface

FUNCTIONS

DCBUF UNIT

DCBUF unit just pipes down the data from external.

It has two usages:

- Bus routing.

- Delay the time to cache will make it easier to inform dispatch.

DCREAD UNIT

Here is the pipeline for load/store/dctl/prefetch

T0		T1		T2		T3

I			DISP 	SEND	INV

F/S 		DISP	SEND	MEM	SYNC && CAST

O/L/UL		DISP	SEND	CHECK	ACTION

Load		DISP	AGU	EXT		WB (registers)

Store		DISP	AGU	WB (data memory)

Prefetch		DISP	SEND	CHECK	PUSHQ

Tag memory chip enable:

- Dctl check valid

- Cacheable load, store or prefetch

- Miss read (allocate new way)

- Invalid

- Miss refill

- Sync or flush hits

Data memory chip enable:

- Dctl check data

- Miss read (allocate new way)

- Cacheable load

- Miss refill

- Dctl invalid cache line

Load and store can access data memory at the same time if they aim at different banks.

Sync action enable:

- Invalid, sync or flush

- Miss refill or eviction.

Cast action enable:

- Sync or flush dirty cache line.

- LRU casts dirty cache line/eviction.

DCMISS UNIT

DCMISS unit can receive at most 2 requests from DCREAD unit when vector load and vector store are dispatched in the same cycle and they fall out of TCM memory. If load misses, the whole pipeline will freeze and no request will come, load will be the last one to DCMISS unit. If store misses, following instructions will continue executing.

Merge cases:

- New store falls into the same cache line with previous store or prefetch.

- New load or prefetch falls into the same cache line with previous store or prefetch.

Cacheable load or store is regarded as 32-bit read for external, if multiple stores update the same cache line, dirty bits and words will record the changes and help to merge changes into returned cache line.

Key words first:

If it’s cacheable load request, tell AXISYNC unit to return key word first, the key word position is hidden in address 4-3. AXISYNC unit will return the request for twice.

- AXISYNC returns the key word first to ensure the pipeline to flow.

- AXISYNC returns the cache line to continue cache refill and other cache operations.

Block conditions:

- If miss queue is full, block dispatch.

- If lock in is miss queue, block dispatch.

- If data is returned, block dispatch to avoid cache pipeline conflict.

If new load request hits returned address, it has no need to push into miss queue and write back immediately.

CRITICAL PATHS

- Tag is read out and participates in comparison and then write to data memory

VECTOR

INTRODUCTION

Vector unit responses for vector calculation: arithmetic, logic, shift, max/min, multiply, reorder, search and accumulator. Column accumulator operation instructions are two-cycle latency, filter instructions are multi-cycle latency, the rest of vector instructions are only single cycle. If you are to confirm which unit the vector instruction belongs to, please refer to micro-arch design sheet.

OVERVIEW

Figure 3-37

Figure 3-38 Vecexec unit overview

INTERFACE

Here is the overview of vector interface.

Figure 3-39

Figure 3-40 Vecexec unit interface

FUNCTIONS

VECALU UNIT

VECALU UNIT support instruction related to addition and subtraction arithmetic operations and mask value operation and resource from generate purpose register. Below is the structure diagram of VECALU UNIT.

Figure 3-41

Figure 3-42 VECALU UNIT structure diagram

In VECALU UNIT, there are several types of instruction:

In VECALU UNIT, there are several types of instruction:

- Normal addition and subtraction arithmetic operation instructions: VADD, VSUB.

- Related to subtraction instructions: VDIFF, VMAX, VMIN, VCMPS, VCMPU.

- Select vector register value instruction: VSEL.

- Replace an element with generate purpose register value instruction: VSETR.

- Broadcast generate purpose register value to target vector register instruction: VBRD.

- Just copy source vector register value to destination register instruction: VCOPY.

VECLSU UNIT

VECLSU UNIT mean vector logic and shift unit, this unit supply logic operation and shift operation. Below is the structure diagram of VECLSU UNIT.

Figure 3-43 VECLSU UNIT structure diagram

In VECALU UNIT, there are several types of instruction:

- Logic operation instructions: VAND, VOR, VCAND, VXOR.

- Shift operation instruction: VSHL, VSHR, VSHRA.

VECMUL UNIT

VECMUL UNIT is designed to VECMUL0 UNIT and VECMUL1 UNIT. VECMUL0 UNIT in the first cycle and the VECMUL1 UNIT is in the second cycle. Below is VECMUL UNIT structure diagrams.

Figure 3-44 VECMUL0 UNIT structure diagram

Figure 3-45 VECMUL1 UNIT structure diagram

VECMUL UNIT instruction can classify into one cycle instructions in VECMUL0 UNIT and two cycle instructions in VECMUL1 UNIT.

- Calculate product of two source vector register instruction and write half result to destination vector register instruction: VMUL.

- Calculate same as VMUL but write full result instruction: V2MUL.

- Calculate 2 multiply by result of V2MUL instruction: V2MULS.

- All of one cycle multiply instruction supply fix-point decimal format.

- Calculate line multiply accumulate instruction: C2MSUM.

- Calculate column multiply accumulate instruction: VCSUM, V2CSUM.

Below is flow diagram of VFILTC and VFILTV instruction:

Figure 3-46

VECSRCH UNIT

VECSRCH UNIT supply find the max/min element across a vector register instruction: VMAXA, VMINA. Below is the structure diagram of VECSRCH UNIT.

Figure 3-47 VECSRCH UNIT structure diagram

VECREORDER UNIT

VECREORDER UNIT is designed for shuffling elements in one or two source vector register then reorder them with specific rules into destination vector register. Below is the structure diagram of VECREORDER UNIT.

Figure 3-48 VECREORDER UNIT structure diagram

In VECREORDER UNIT, there are several types of instruction:

- Select some elements from two source vector register and concatenate them to a full vector register instruction: VCONCAT.

- Divide elements of a source vector register into several groups, then select either the beginning or ending element of each group instruction: VEXT.

- Group same as VEXT, but only choose the beginning or ending element of each group instruction: VEXTB.

- Group same as VEXT, but choose odd/even groups instruction: VDILV.

- Copy source vector register except one element instruction: VTAIL.

- Groups same as VECT, then replace each group several times until get a full vector register instruction: VSPREAD.

- Duplicate each element in source vector register from the first one several times until get a full vector register instruction: VDUPL.

- Interleave copy the element of two source vector register until get full destination vector register instruction: VILV.

- Insert the first element of one source vector register somewhere in destination vector register instruction: VINS.

VECOTHER UNIT

VECOTHER UNIT supply some special instructions, below is the structure diagram of VECOTHER UNIT.

Figure 3-49 VECOTHER UNIT structure diagram

In VECOTHER UNIT, there are several types of instruction:

- Select an element from source vector register then send it to generate purpose register instruction: RSETV.

- Pack word/halfword length element from source vector register to halfword/byte length element instruction: VPACK.

- UNPACK halfword/byte length element from source vector reg to word/halfword length element instruction: VUNPACK.

- Translate a generate purpose register value to vector mask register instruction: MSETR.

CRITICAL PATHS

- Result mux is traditional critical path.

- VECREORDER UNIT generate select mask is also critical path.

- Fix-point decimal multiplication execution is critical path.

- Accumulate register multiply-accumulate operation is critical path.

WRITE BACK

INTRODUCTION

WRITE BACK unit responses for collecting different kinds of results and exceptions from subunits and interrupts from external. If exceptions or branch taken is found, restart the pipeline. Our pipeline is 3-way width, so WRITE BACK unit has some same cycle qualification jobs to do. If instructions are confirmed to complete, write back the results to architecture registers in DISPATCH unit. Besides, some global control SPRs are updated in WRITE BACK unit.

OVERVIEW

Figure 3-50 write back overview

- Most of Write back interfaces are bus, WRITE BACK unit collects all results and sends them to the right architecture registers.

- Same cycle qualification contains branch taken cancel and exception cancel. The slot number helps to know the instruction order within one dispatch package. If two-cycle instructions write back in this cycle, they are older than single-cycle ones.

INTERFACE

Figure 3-51 write back unit interface

FUNCTIONS

CYCLE QUALIFICATION

All instructions should get qualified by branch flush, exceptions. Two-cycle instruction is older than one-cycle instruction if they arrive at their WB stage at the same time. If instructions have the same latency, dispatch-slot helps to check which is older. The special case is that update store or load occupies duel pipeline and if memory access pipeline finds exceptions, the other part should be flushed.

Integer 0 complete condition:

- No pending interrupt

- No execution or debug exception

- No load missed

- No older branch taken

- No older load or store exceptions

Integer 1 complete condition:

- No pending interrupt

- No SWI/TRAP or debug exception

- No load missed

- No older load, store or integer exceptions

load complete condition (no exception or flush happens with load at the same cycle):

- No pending interrupt

- No load or debug exception

- No load missed

Store complete condition:

- No pending interrupt

- No store or debug exception

- No load missed

- No older branch taken

- No older integer exception

Vector complete condition:

- No pending interrupt

- No debug exception

- No load missed

- No older branch taken

- No older load, store or integer exception

Dctl complete condition (dctl always stays at the first slot):

- No pending interrupt

- No dctl or debug exception

- No load missed

ARCHITECTURE UPDATE

Pick the right complete and write back to corresponding register file. Perhaps the most important thing is to check the same destination writes and pick out the newer one.

- Load and integer may write to the same GPR, ADR, BTR.

- Load and vector may write to the same VCR.

- Integer0 and integer1 may write to the same GPR, ADR, BTR, CF.

- Integer 1 and vector may write to the same ACR.

SPR UPDATE

Critical SPRs like CPUC, PSC, exception control and recovery related SPRs are updated in WRITE BACK unit. Config, debug, timer and performance counter are updated in the subunits where their logics are close to.

To simplify SPR decode logic, the SPRs which are accessed in the same unit stay close to each other.

FFT related SPRs are special, strictly speaking, FFTW and FFTD are memory. If read or write to FFTW and FFTD is defined as load and store instruction, the latency may be long.

Only integer 0 can read and write SPR, the exception check logic locates at integer 0 and read/write logic locates at INTSPR unit.

If you are to check detailed SPR description, please refer to ISA document.

EXCEPTION HANDLER

Dispatch exception:

- Illegal instruction exception

- Permission exception

- Debug exception

Execution exception:

- SPR undefined exception

- SPR restart exception

- Divide-by-zero exception

- SWI/TRAP exception

- Timer exceptions: WATCH-DOG, COUNT-DOWN, TIMER BASE

- Load exceptions: UNALIGN, PERMISSION, RESTART, DEBUG, CROSS-DOMAIN

- Store exceptions: UNALIGN, PERMISSION, RESTART, DEBUG, CROSS-DOMAIN

- Dctl exceptions: UNALIGN, PERMISSION, RESTART, DEBUG

External interrupts

If you are to check which pending bit they will set, please refer to ISA document.

PC SWAP LOGIC

To save the resources, not all instructions carry their pc in local pipeline, this is kind of wasteful. DISPATCH unit will pipe down the package pc to WRITE BACK unit and generate all instruction PCs.

In current design, two-cycle instructions can write back with one-cycle instructions. 3 PCs are latched for two-cycle instructions. If only two-cycle instructions write back, pick last valid pc from latched version. If one-cycle instructions write back, pick last valid pc from current input ones. Another 3-bit vector is latched to record the completed instructions.

CRITICAL PATHS

- Write back to dispatch register files.

- Flush signal to qualify store and dctl.

TIMER

INTRODUCTION

TIMER unit consists of a 64-bit timer-base register that is incremented at the clock-frequency divide by 64, and a 32-bit count-down registers that are decremented at the frequency. Timer is 64-bit width to avoid overflow quickly.

OVERVIEW

Figure 3-52 timer overview

FUNCTIONS

When the TCD0 counts down to 0, or when increment of the TIM generates a carry-out from certain bit position, it is possible to cause a timer exception. These are controlled and recorded by the time-base control and status register (TIMC) and the count-down control and status registers (TCD0C).

INTSPR

INTRODUCTION

INTSPR unit is responsible for SPR read and write, containing performance counter which is also defined as a kind of SPR. It’s not friendly for layout to read all SPRs from everywhere, so INTSPR unit collects SPRs in a small memory. When dispatch unit finds a SPR read, it reads INTSPR memory and returns the value to write back unit. When SPR write is confirmed to complete, write back unit writes INTSPR memory. In trap and exception case, write back unit also update SPRs, which means SPR write has higher priority than SPR read.

OVERVIEW

Figure 3-53 intspr overview

INTERFACE

Figure 3-54 intspr unit interface

FUNCTIONS

SPR MEMORY ACCESS

Chip enable condition:

- RSETSPR and its index ranges from 16 to 128

- SPRSETR

- Exception will save XR, XS, XI, XA automatically.

It’s implemented by 32x64 memory with write mask. Exception can write at most 4 SPRs and SPRSETR will write only half line of the memory.

Write has higher priority than read.

Here is the pipeline of read and write:

T0		T1		T2		T3

SPRSETR		DISP	CHECK	WR

RSETSPR				DISP	CHECK	WB

For global control SPRs, block all following instructions until it updates the SPR which will avoid the memory port conflict. To make its control simple, all SPR writes will block dispatch.

PERFORMANCE COUNTER

To make it configurable and scalable, we define performance counter as SPRs from 128-191 which means we can track at most 64 events from all subunits of DSP. We divide them into 4 group according to function and position.

Fetch events:

Monitor cache, ITCM and BP table behaviors.

Dispatch events:

Monitor dispatch bandwidth, all kinds of stall and optimization behaviors.

Load/store events:

Monitor cache, DTCM and memory access behaviors.

Write back events:

Monitor all kinds of completed instructions, exceptions, interrupts.

If you are to review detailed events description, please refer to ISA document.

Each group has 3 counters to record at most 3 events. It’s easy to add more events to each group and make them share the same counter resources.

Performance counter write will allocate and initialize the performance counter. Then the counter will increment when sensitive events are toggled.

Performance counter read will read out the value and release the counter until it’s qualified in WB stage.

When the number of pending events is greater than 3 in a group, the first events will be flushed automatically.

SPR BYPASS

RSETSPR reads out value in EX stage and pipe it down to WB stage. RSETSPR does not support WB to EX forwarding. The mux to select RSETSPR result is in EX+1 stage and EX+2 can forwarding the RSETSPR result to EX.

CRITICAL PATHS

None

COREIO

INTRODUCTION

COREIO contains COREIN unit and COREOUT unit, which is the interface between DSP and external (AXI/PXI/FFT). Instruction cache request and data cache request will be pushed into two FIFOs (AXI FIFO and PXI FIFO) in order according to their address. If AXISYNC unit pulls request and AXI FIFO is not empty, COREOUT unit will output request to AXISYNC unit to check cache coherency and then send to AXI bus. If XREQQ unit pulls request, PXI FIFO is not empty and PXI load has entry, COREOUT unit will output request to XREQQ unit. COREIN unit receives eviction and data from external.

OVERVIEW

Figure 3-55 coreout overview

- Cast request will be pushed into PXI queue if its address fall into PXI region, the 256-bit data will occupy 2 entries to save entry size.

- Sync request will pipe to AXISYNC unit and operate directory directly.

FUNCTIONS

COREIN TIMING ISSUE

Here is the pipeline for eviction and refill:

T0		T1		T2		T3		T4			T5

Eviction	COREIN	BUFFER	READ	CHECK	CAST/SYNC	COREOUT

Refill	COREIN	BUFFER	READ	REFILL	COREOUT

CHECK

DISP		EXE		WR

DISP	EXE		WR

DISP	EXE		WR

Dispatch hold:

Store or dctl will occupy cache port for two cycles which is easier to have conflict with eviction or refill. From the pipeline point of view, an external response will cause dispatch to stall 3 cycles (T0, T1, T2).

Refill caused stall:

Besides, if refill is received in T0, then the next refill or eviction can not enter COREIN unit in T1 because of cache port conflict.

Eviction caused stall:

If eviction is received in T0, then the next refill or eviction cannot enter COREIN unit until the eviction is completed.

If miss queue has valid miss requests, dctl cannot be sent which will ensure that dctl will send to cache until refill is completed.

The last issue is that how does dctl/refill execute with eviction:

T0		T1		T2		T3		T4		T5

Sync/flush	READ		COREOUT

Sync/flush	DISP		EXE		CHECK	READ	COREOUT

Eviction				COREIN	BUFFER	READ	CHECK	CAST/SYNC	COREOUT

T0		T1		T2		T3		T4		T5

Refill		COREIN	BUFFER	CHECK	REFILL	COREOUT

Refill						COREIN

Eviction										COREIN

At most 2 cast requests are on the way when eviction tries to enter COREIN unit.

LOCK TIMING ISSUE

When lock/unlock/open is executed before eviction arrives:

T0		T1		T2		T3		T4		T5

Lock	DISP		EXE		CHECK	ACTION

Eviction				COREIN	BUFFER	READ	CHECK	CAST/SYNC	COREOUT

To read the latest cache line property, eviction should read lock property in T4 cycle which is the reason why miss request read lock in refill cycle.

CRITICAL PATH

None

SYNTHESIS

28 nm technology from TSMC.

| Partition | Area | Power | Timing | Cells |
| --- | --- | --- | --- | --- |
| IFETCH |  |  |  |  |
| DISPATCH |  |  |  |  |
| DATXFER |  |  |  |  |
| DCACHE |  |  |  |  |
| VECTOR |  |  |  |  |
| INTEGER0 |  |  |  |  |
| INTEGER1 |  |  |  |  |
| WRTBK |  |  |  |  |

PERFORMANCE

| SPEC Item | benchmark | bottleneck | optimization | Gain |
| --- | --- | --- | --- | --- |
| DHRYSTONE |  |  |  |  |

OPTIMIZATION

| Feature | Description | Performance gain |
| --- | --- | --- |